715.868 Acres

A certain parcel of land in Kentucky lying in Trigg County southwest of the city of Cadiz more particularly described as follows:

Unless stated otherwise, any monument referred to herein as an iron pin set is a 1/2” rebar 18” in length with plastic cap stamped “DAVIDSON PLS 4145." All tree monuments have three hack marks. All bearings and coordinates are based on Kentucky Single Zone Coordinate System by GPS, using Geoid 2012b.

Beginning at an iron pin found ½” rebar, PLS 4145 at the southeast right of way of Highway 68 (Transportation Cabinet, Department of Highways Right of way varies as per Deed Book 283, Page 48) and the southwest right of way of White Tail Creek Rd (30' Right of way as per KRS 178), lying approx. S 41°30' W  504' for the centerline intersection of Highway 68 and White Tail Creek Rd, and having a coordinate of N:3460054.37, E:4289059.83.  Thence with said right of way of White Tail Creek Road a curve turning to the left having an arc length of 45.39', a radius of 79.91', a chord bearing of S 15°54'36" W, and a chord length of 44.78' to an iron pin found a corner to Sean Hunter McElwain (Deed Book 274, Page 47);

thence with Sean Hunter McElwain for 2 calls S 42°54'33" W, 51.88' to an iron pin found (1/2” rebar, PLS 4145); thence S 69°55'59" E, 103.99' to an iron pin set I the aforementioned right of way of White Tail Creek Rd;

thence with said right of way for several calls S 72°39'41" E, 168.74'; thence S 67°00'10" E, 171.02'; thence S 71°51'01" E, 148.46'; thence S 66°00'59" E, 141.55'; thence S 70°29'16" E, 86.14'; thence S 74°12'07" E, 124.64' to an iron pin set; thence S 61°16'45" E, 79.38'; thence S 44°10'23" E, 91.48';  thence S 63°57'41" E, 163.18'; thence S 43°07'53" E, 33.52';  thence S 22°07'16" E, 74.42'; thence S 43°44'08" E, 49.40'; thence S 69°34'03" E, 64.10'; thence S 80°10'26" E, 62.93'; thence S 88°30'46" E, 143.80'; thence S 74°39'44" E, 42.57'; thence S 59°32'38" E, 69.40'; thence S 79°43'14" E, 109.57' to an iron pin set; thence S 63°43'50" E, 32.12'; thence S 47°14'01" E, 65.56';thence S 21°00'01" E, 118.68'; thence S 41°53'59" E, 49.39';

thence with said right of way and then Pauline McIntosh and Diana McIntosh (Deed Book 148, Page 237) S 65°00'01" E, 192.42' to a 24” red oak;

thence with Pauline McIntosh and Diana McIntosh (Deed Book 148, Page 237) N 78°13'41" E, 108.86' to an iron pin set in the aforementioned right of way of White Tail Creek Rd;

thence with said right of way for several calls N 79°30'22" E, 154.04'; thence S 57°39'44" E, 157.51'; thence S 40°29'47" E, 37.20'; thence S 06°41'20" W, 47.15' to an iron pin set; thence S 31°44'05" W, 71.41'; thence S 14°54'52" W, 68.13'; thence S 17°51'18" E, 61.74'; thence S 51°39'12" E, 83.56'; thence S 61°36'31" E, 178.59'; thence S 55°19'56" E, 284.23'; thence S 63°09'52" E, 56.94'; thence S 72°43'32" E, 302.04'; thence S 79°38'54" E, 64.89'; thence S 87°53'49" E, 62.61' to an iron pin set a corner to Alvin "Butch" McElwain and Connie McElwain (Deed Book 172, Page 247);

thence with Alvin "Butch" McElwain and Connie McElwain (Deed Book 172, Page 247) for 3 calls S 11°39'58" W, 79.76' to an iron pin found (1/2” rebar); thence S 65°24'22" E, 199.93' to an iron pin set;  thence S 80°45'50" E, 364.46' to an iron pin found (1/2” rebar) a corner to Knight and Hale Game Properties, Limited (Deed Book 174, Page 288);

thence with Knight and Hale Game Properties, Limited (Deed Book 174, Page 288)  for 7 calls S 32°32'53" W, 4.99' to an iron pin set; thence S 80°20'16" E, 424.78' to an iron pin found (1/2” rebar, PLS 4145); thence S 60°44'51" E, 168.16' to an iron pin found (1/2” rebar, PLS 4145); thence S 52°27'46" E, 422.67' to an iron pin found (1/2” rebar, PLS 4145); thence S 43°10'50" E, 383.55' to an iron pin found (1/2” rebar, PLS 4145); thence S 44°49'41" E, 342.03' to an iron pin found (1/2” rebar, PLS 4145); thence S 41°49'11" E, 282.54' to an iron pin found (1/2” rebar, PLS 4145); thence S 72°09'18" E, 432.98' to a iron pin found (1/2” rebar, PLS 1532) a corner to William C. Stallons and Penny C. Stallons (Deed Book 278, Page 628) and Sue Anna King and Marvin C. Calhoun and Dale Thomas Calhoun (Deed Book 141, Page 758);

thence with Sue Anna King and Marvin C. Calhoun and Dale Thomas Calhoun (Deed Book 141, Page 758) for 6 calls S 72°32'32" W, 395.96' a 48” Sycamore; thence S 49°24'14" W, 543.04' to a iron pin found (1/2” rebar, PLS 1532); thence S 54°45'37" W, 2214.21' an iron pin found (3/8” rebar); thence S 01°15'20" W, 895.60' to an iron pin found (3/8” rebar); thence N 88°41'20" E, 578.35' to an iron pin set; thence S 00°51'54" W, 885.35' to an iron pin found (3/8” rebar) a corner to Bobby D. Wroten and Dianne C. Wroten (Deed Book 275, Page 49);

thence with Bobby D. Wroten and Dianne C. Wroten (Deed Book 275, Page 49) S 09°19'12" E, 916.48' to a iron pin found (1/2” rebar, PLS 3210);

thence with Bobby D. Wroten and Dianne C. Wroten (Deed Book 275, Page 49) and then Bobby D. Wroten and Dianne C. Wroten (Deed Book 286, Page 661), and then Robert Hale Upton and Jo Ann Ross Upton and Harold Holmes and Susan Upton Holmes (Deed Book 128, Page 329) S 89°01'46" W, 3555.89' to an iron pin set;

thence with Robert Hale Upton and Jo Ann Ross Upton and Harold Holmes and Susan Upton Holmes (Deed Book 128, Page 329) N 09°14'10" E, 1032.98' to a stone found;

thence with Robert Hale Upton and Jo Ann Ross Upton and Harold Holmes and Susan Upton Holmes (Deed Book 128, Page 329) and then Herdon Cemetery Rd (County Right of way) and then Mark Douglas and Robin Douglas (Deed Book 257, Page 515) and then Eva Jo Ray (Deed Book 242, Page 631) N 09°11'22" W, 1402.59' to an iron pin found (1/2” rebar);

thence with Eva Jo Ray (Deed Book 242, Page 631) for 3 calls S 88°43'47" W, 56.07' to an iron pin found (1/2” rebar); thence N 15°48'43" E, 437.24' to a ¾” iron rod; thence N 63°35'17" W, 755.19' to a point (referenced by an iron pin found (1/2” rebar, PLS 3536), lying S 68°57'42" W  27.22' form said point;

thence with Eva Jo Ray and then Mark Douglas and Robin Douglas (Deed Book 257, Page 515) S 81°25'43" W, 438.65' to an iron pin found (1/2” rebar);

thence with Mark Douglas and Robin Douglas (Deed Book 257, Page 515) for 4 calls N 02°43'40" E, 486.02' to an iron pin found (1/2” rebar); thence N 71°48'04" W, 174.46' to an iron pin found (1/2” rebar); thence N 01°24'20" E, 768.54' to an iron pin found (1/2” rebar); thence S 89°58'59" W, 726.09' to a 1/2" iron pipe found a corner to Kenneth Slocum and Beverly Slocum (Deed Book 169, Page 67);

thence with Kenneth Slocum and Beverly Slocum (Deed Book 169, Page 67), then Arnold Chulpsa and Verna Chlupsa (Deed Book 166, Page 630), then Arnold Chulpsa and Verna Chlupsa (Deed Book 166, Page 630, Plat Cabinet A, Slide 197, Lot 10), then Jeffrey Wigand and Marianne Wigand (Deed Book 210, Page 100, Plat Cabinet A, Slide 197) N 09°22'16" E, 2387.14' to an iron pin found (1/2” rebar, PLS 3210) a corner to Ashley Plymale and Maria Plymale (Deed Book 254, Page 705, Deed Book 272, Page 295);

thence with Ashley Plymale and Maria Plymale (Deed Book 254, Page 705, Deed Book 272, Page 295) for 10 call S 83°35'43" E, 478.92' Iron Pin Found (1/2" rebar, PLS 3210);thence S 83°29'28" E, 19.50' Iron Pin Found (1/2" rebar, PLS 3210);thence N 30°54'16" E, 95.57' Iron Pin Found (1/2" rebar, PLS 3210);thence N 37°19'59" E, 129.86' Iron Pin Found (1/2" rebar, PLS 3210);thence N 04°07'04" E, 76.87' Iron Pin Found (1/2" rebar, PLS 3210);thence N 26°53'19" E, 266.01' Iron Pin Found (1/2" rebar, PLS 3210);thence N 64°56'10" W, 4.51' Iron Pin Found (1/2" rebar, PLS 3210) to a point (referenced by an iron pin found (1/2” rebar, PLS 3210), lying S 20°01'01" W  37.69' from said point) 15’ west of the centerline of a paved drive a corner to Harold Knight, Judy Knight, David Hale & Pam Hale (Deed Book 319, Page 698);thence with Harold Knight, Judy Knight, David Hale & Pam Hale (Deed Book 319, Page 698) and meandering 15’ west and parallel to said centerline of paved drive for 5 calls N 15°26'22" E, 39.44'; N 00°42'11" W, 41.85'; N 15°49'38" W, 39.39'; N 02°44'30" W, 60.63'; N 14°13'27" E, 173.40' to a point (referenced by an iron pin found (1/2” rebar, PLS 3010, lying S 79°28'15" E  30.06' from said point) in the aforementioned right of way of KY Highway 68;

Thence meandering said right of way for  the remaining 3 calls S 79°28'15" E, 46.82’;  N 66°28'20" E, 201.70';  N 45°35'27" E, 255.01', to the point of beginning, having an area of 715.868 Acres according to this survey performed by Davidson Land Surveying,  270-202-2236, claydavidsonpls@gmail.com, William Clay Davidson (PLS No. 4145, issued May 20, 2016). Dated: 10/8/2021, Job No. 20-126.

Source of Title:  A portion of Deed Book 313, Page 510